If you would like a laminated A5 version with your boat details printed please email me.

This Mayday card is for you to download and modify.  Click ‘File’ and ‘Save as’ Word document. Keep it by your VHF.

Note that VHF DSC procedures vary between manufacturers, check the instructions for your own set.  All use is at your own discretion.

MAYDAY – EMERGENCY
VHF RADIO PROCEDURE - DSC
Use only when there is imminent and grave
danger to life or vessel

BOAT NAME:

MMSI:

CALL SIGN:

Check radio is switched on at PANEL and SET.

Lift the cover on the red DISTRESS button and press the button through the countdown.

Wait 15 seconds. If there is no reply, press the transmit button on the handset and say SLOWLY:

- MAYDAY,  MAYDAY,  MAYDAY

- THIS IS . . . Yacht - Name -  3 times

- MAYDAY . . . Yacht  - Name     MMSI………………

- MY POSITION IS . . . latitude and longitude, or a true bearing and distance from a known point.  Don’t guess.

- I AM .  . . state the nature of distress

- I REQUIRE IMMEDIATE ASSISTANCE

- I HAVE . . . (number) of people on board

- INFORMATION . . .   such as liferaft, flare fired, etc

- OVER  -  this means 'reply to me’

Release Transmit button.

Listen on Channel 16.  If you hear nothing repeat the call.

MAYDAY – EMERGENCY
VHF RADIO PROCEDURE – NON DSC
Use only when there is imminent and grave
danger to life or vessel

BOAT NAME:

CALL SIGN:

Check radio is switched on at PANEL and SET.

Press the transmit button on the handset and say SLOWLY:

# MAYDAY,  MAYDAY,  MAYDAY

- THIS IS . . . Yacht - Name -  3 times

- MAYDAY . . . Yacht  - Name

- MY POSITION IS . . . latitude and longitude, or a true bearing and distance from a known point.  Don’t guess.

- I AM .  . . state the nature of distress

- I REQUIRE IMMEDIATE ASSISTANCE

- I HAVE . . . (Number) of people on board

- INFORMATION . . .   such as liferaft, flare fired, etc

- OVER  -  this means 'reply to me’

Release Transmit button.

Listen on Channel 16.  If you hear nothing repeat the call.